Hybrid Machine Learning-Based Prediction and Optimization of Composite Material Properties

1. Introduction

Composite materials are widely used in engineering applications due to their enhanced mechanical properties such as tensile strength, flexural strength, and impact resistance. However, traditional experimental approaches for material design are often time-consuming, costly, and resource-intensive.

To overcome these limitations, this study proposes a hybrid machine learning approach that combines simulated and real experimental data to accurately predict and optimize composite material properties.

2. Dataset Description

Two types of datasets were used in this study:

Simulated Dataset

- Large dataset (~1000 samples)

- Generated under ideal conditions

- Smooth and noise-free

Real Experimental Dataset

- Smaller dataset (~100–150 samples)

- Contains noise and real-world variations

- Represents practical material behavior

Hybrid Dataset

To leverage the advantages of both datasets, they were combined to form a hybrid dataset, enabling the model to learn both general patterns and realistic behavior.

3. Data Preprocessing

The following preprocessing steps were applied:

- Removal of missing values

- Conversion of all features to numeric format

- Alignment of input and output variables

Input Features:

- Snake Plant wt (%)

- Fish Scale wt (%)

- Epoxy wt (%)

- Layer Thickness (mm)

- Curing Temperature (°C)

- Curing Time (hr)

- Pressure (MPa)

Target Variables:

- Tensile Strength (MPa)

- Flexural Strength (MPa)

- Impact Strength (kJ/m²)

4. Hybrid Learning Strategy

A key challenge in this study was balancing data realism and data volume:

- Simulated data → large but less realistic

- Real data → realistic but limited

Solution:

A hybrid dataset was created and a weighted learning approach was applied:

- Real data → weight = 3

- Simulated data → weight = 1

This ensures the model prioritizes real-world behavior while still benefiting from a larger dataset.

5. Exploratory Data Analysis (EDA)

Distribution Analysis

The distributions of tensile, flexural, and impact strength show generally normal behavior with the presence of some high-value outliers.

Boxplot Analysis

- Flexural strength exhibits the highest variability

- Tensile strength shows moderate spread

- Impact strength remains relatively lower

Correlation Analysis

The correlation heatmap reveals:

- Strong positive correlation between tensile and flexural strength (≈0.82)

- Snake plant fiber content positively affects all mechanical properties

- Fish scale content negatively affects strength

- Curing time and temperature show moderate positive influence

6. Model Development

Several machine learning models were implemented:

- Random Forest

- Extra Trees

- XGBoost

- LightGBM

- CatBoost

All models were trained using the hybrid dataset.

7. Model Evaluation

The models were evaluated using:

- Mean Absolute Error (MAE)

- Root Mean Square Error (RMSE)

- R² Score

Results:

The Extra Trees Regressor achieved the best performance:

- Lowest MAE

- Lowest RMSE

- High prediction accuracy

8. Model Performance Analysis

Actual vs Predicted

The predicted values closely align with actual values, indicating strong model performance.

Residual Analysis

Residual plots show random distribution around zero, confirming:

- No systematic error

- No significant bias

- Good generalization capability

9. Feature Importance

Feature importance analysis reveals:

- Epoxy wt (%) → most influential

- Layer thickness

- Snake plant wt (%)

This indicates that material composition plays a dominant role in determining mechanical performance.

10. Optimization

To identify optimal material designs:

- 5000 random design combinations were generated

- The trained model predicted their performance

Scoring Function:

Score=Tensile + Flexural + Impact

Top-performing designs were selected based on this score.

11. Feasible Design Filtering

To ensure practical applicability, constraints were applied:

- Tensile Strength ≥ 60 MPa

- Flexural Strength ≥ 85 MPa

- Impact Strength ≥ 45 kJ/m²

From the optimized results, the top 5 feasible designs were selected.

Key Observations:

- Higher snake plant fiber content improves performance

- Lower fish scale content leads to better strength

12. Results Summary

- Hybrid dataset improved model learning

- Extra Trees model achieved best performance

- Optimization successfully identified high-performance designs

- Feasible filtering ensured practical applicability

13. Conclusion

This study presents a hybrid machine learning framework for predicting and optimizing composite material properties.

The proposed approach:

- Reduces experimental cost and time

- Improves prediction accuracy

- Enables data-driven material design

- Produces practically feasible optimized solutions

14. Future Work

Future improvements may include:

- Incorporating deep learning models

- Expanding real experimental datasets

- Multi-objective optimization

- Deployment as an industrial decision-support tool

Final One-Line Summary

This work demonstrates how hybrid machine learning can effectively bridge the gap between simulated and real data to enable accurate prediction and optimization of composite materials.